# Standard Operating Procedure (SOP)

## Printer Setup for MPCM

Document Reference: CEPT
Equipment: Dot Matix Printer
Version: 1.0.1
Date: July 2025

## Purpose

This SOP provides step-by-step instructions for setting up and configuring the Printer for MPCM receipt printing operations.

## Scope

This procedure applies to all personnel responsible for installing, configuring, and maintaining MPCM receipt printing systems

## Required Materials

- Dot Matrix Printer

- USB Cable

- MPCM sticker paper/receipt roll

- ReceiptPrinterService.msi installation file

- Computer system with administrative privileges

## Procedure

### Physical Setup and Connection

Initial Connection

- Connect the Dot Matrix printer to the computer system using the provided USB cable

- Power on the printer using the main switch

Removing of additional printers which are connected to System

- Go to Control panel

- Click on Hardware and Sound

- Click on Devices and Printer

- Click on Printers & Scanners

- Remove all unnecessary printers available in above screen shot. You can keep Microsoft print to PDF and network printers

- Install required Dotmatrix software

- Download the MPCM Printer software developed by CEPT from  below mentioned link as on when updated

https://drive.google.com/drive/folders/1YdHpJLRzi7R946qxP8LB7Wj-3irn7mTl

- Do the required paper settings and margins for MPCM Receipt printing

- Go check the status of printer if Offline ,  switch on printer then the offline will disappear

### Printer Server Configuration

Form Creation

- Access Printer Server Settings

- Create a new form named "MPCM Receipt"

•	Click on Create New form

•	Rename the Form as MPCM Receipt

•	Configure the form dimensions according to MPCM receipt specifications

- Give margins as shown below and click on Save Form

  - Paper Size: Width: 17.00cm, Height: 5.10cm

  - Printer Area Margins: Left: 0.02cm, Right: 0.02cm, Top: 0.02cm, Bottom: 0.02cm

  - Units: Metric

- Ensure margins are optimized for MPCM receipt format

- Click "Save Form" to save the configuration

- Go to control panel – >Printers - >  Right click on Default dot matric Printer

- Click on printer properties – The screen will appear like this

- Click on Advance – The screen will appear this . Then change the following

Right Click on the Printer- Properties-Advanced-Printing Defaults-Advanced • Select Paper Size- MPCM Receipt.

- Again goto Device and printer in control panel and Right click on Printer and got to Printer Properties and click o  Printing preferences   and Check for Paper Source.

Step 2: Paper Loading

- Feed the MPCM sticker paper through the spooler roller ONLY

- Ensure the MPCM sticker paper is properly locked with the roller mechanism

- Verify that the paper is seated correctly and securely

Phase 2: Driver Installation and Configuration

Step 3: Driver Setup

- Check if printer drivers are installed on the system

- If not installed, install the appropriate printer drivers

- Set this printer as the default printer in system settings

Step 4: Initial Testing

- Perform a test print to verify basic printer functionality

- Ensure print quality and alignment are satisfactory

### Phase 3: Service Installation

Step 5: Service File Preparation

- Download the ReceiptPrinterService.msi file

- Copy the file to either:

  - Desktop, OR

  - D: drive root directory

Step 6: Service Installation

- CRITICAL: Install the ReceiptPrinterService under Administrator privileges

- Right-click on the .msi file and select "Run as administrator"

- Follow the installation wizard prompts

Step 7: Service Verification

- After installation, verify that the service is running:

  - Open Windows Services (services.msc) in command prompt

  - Locate the ReceiptPrinterService in the services list

  - Verify the service status shows as "Running"

Step 8: Service Configuration

- In the Services list, locate the ReceiptPrinterService

- Right-click and select "Properties"

- Navigate to the "Log On" tab

- Configure the service to "Run as administrator"

- Apply changes and restart the service if necessary

### Phase 4: Printer Server Configuration

Step 9: Form Creation

- Access Printer Server Settings

- Create a new form named "MPCM Receipt"

•	Click on Create New form

•	Rename the Form as MPCM Receipt

•	Configure the form dimensions according to MPCM receipt specifications

Step 10: Margin Settings

- Apply page margin settings as specified in the reference documentation:

  - Paper Size: Width: 17.00cm, Height: 5.10cm

  - Printer Area Margins: Left: 0.02cm, Right: 0.02cm, Top: 0.02cm, Bottom: 0.02cm

  - Units: Metric

- Ensure margins are optimized for MPCM receipt format

- Click "Save Form" to save the configuration

### Phase 5: Final Setup and Testing

Step 11: Paper Management

- Always maintain receipt paper in roll format within the printer

- Ensure paper is properly fixed in the spokes of the spooling roller

- Verify paper alignment and tension

Step 12: Automatic Operation Verification

- Test the automatic sensor functionality

- Confirm that the sensor stops after printing one receipt

- Verify that the paper positioning allows for easy manual tearing

- Test the tear-off mechanism for clean separation

## Visual Reference Guide

### Key Components Identification

- Control Panel: Located on the front of the printer with status LEDs (Power, Draft, Roman, Sans Serif) and control buttons (Tear Off, LF/FF, Load/Eject, Paper Out)

- Paper Feed Mechanism: Internal rollers and spooler system visible when printer cover is open

- Tractor Feed: Blue clips on sides for securing continuous form paper

- USB Connection: Located on the back/side of the printer for computer connectivity

### Proper Paper Alignment

- Continuous form paper should feed smoothly through the tractor mechanism

- Perforations should align properly for clean receipt separation

- Paper tension should be adequate but not too tight to avoid jams

### Form Configuration Settings

- Form Name: "MPCM Receipt"

- Metric units preferred for precise measurements

- Minimal margins (0.02cm) for maximum print area utilization

- Standard receipt dimensions optimized for MPCM requirements

Before completing the setup, verify the following:

- [ ] Printer physically connected and powered on

- [ ] MPCM paper properly loaded and locked

- [ ] Printer drivers installed and set as default

- [ ] Test print completed successfully

- [ ] ReceiptPrinterService installed under admin privileges

- [ ] Service running and configured properly

- [ ] MPCM Receipt form created in printer server

- [ ] Margin settings applied correctly

- [ ] Paper roll properly positioned in spooling roller

- [ ] Automatic sensor functioning correctly

- [ ] Paper tear mechanism working smoothly

## Troubleshooting

### Common Issues and Solutions

Issue: Printer not recognized by system

- Solution: Reinstall printer drivers and check USB connection

Issue: Paper jam or feeding problems

- Solution: Ensure paper is loaded through spooler roller only; do not use manual roller knob when printer is on

Issue: Service not running

- Solution: Verify admin installation, check service configuration, restart service

Issue: Incorrect print formatting

- Solution: Verify MPCM Receipt form settings and margin configurations

Issue: Sensor not stopping after one receipt

- Solution: Check paper positioning in spooling roller spokes